TITLE: Root Cause Analysis — Production Line 3 Unplanned Stoppage

ORGANIZATION: Northwind Analytics, Inc.

DEPARTMENT: Manufacturing Operations

SENSITIVITY: Internal

VERSION: 1.0

EFFECTIVE_DATE: 2026-06-18

_______________________________________________________________________________________________

# Root Cause Analysis — Production Line 3 Unplanned Stoppage

This report documents the root cause analysis for an unplanned stoppage on Production Line 3 at the Northwind Analytics, Inc. manufacturing facility, resulting in approximately 5.5 hours of lost production time.

# 1. Incident Summary

| Field | Detail |
| --- | --- |
| Date/Time | June 12, 2026, 02:14 – 07:41 (night shift) |
| Line Affected | Production Line 3 (sub-assembly) |
| Production Impact | 5.5 hours lost, approx. 1,240 units below plan |
| Severity | Sev-2 — single line impact, no safety incident |

# 2. Timeline

02:14 — Line 3 conveyor motor trips on overcurrent fault; automatic stop triggered.

02:20 — Night shift supervisor logs the incident and pages on-call Maintenance.

02:55 — Maintenance technician arrives on-site, begins diagnostics.

04:30 — Root fault identified as a failed motor bearing; replacement part not in local spares inventory.

04:35 — Part sourced from Line 5 spares (compatible model) with Plant Manager approval.

07:20 — Replacement completed, line restarted for test cycle.

07:41 — Line 3 confirmed stable at full rate, incident closed.

# 3. Root Cause

The 5-Whys analysis traced the bearing failure to an overdue preventive maintenance (PM) interval: the Line 3 conveyor motor's scheduled bearing inspection (every 2,000 operating hours) was last completed 2,340 hours prior, 340 hours past due, due to a scheduling gap when the PM calendar was migrated to the new CMMS system in March 2026.

# 4. Contributing Factors

The compatible replacement bearing was not stocked in Line 3's local spares kit, adding approximately 2 hours to the repair versus if it had been on hand.

The CMMS migration in March 2026 did not carry over 3 of the 40 active PM schedules plant-wide, of which this was one; the gap was not caught by the migration validation checklist used at the time.

# 5. Corrective Actions

| Action | Owner | Due Date |
| --- | --- | --- |
| Audit all PM schedules migrated to CMMS for completeness | Maintenance Manager | 2026-06-30 |
| Add Line 3 bearing kit to local spares inventory | Plant Supply Chain | 2026-06-25 |
| Add migration-completeness check to CMMS change process | IT/OT Systems | 2026-07-10 |

# 6. Sign-off

Reviewed by Plant Manager and Maintenance Manager. Corrective actions tracked in the plant CMMS action log; closure requires verification by Maintenance Manager.